Field installable FO connector SC OM3 3mm

for field termination of fiber optic cables without the need for splicing, curing or polishing.

Connector ferrule holds factory-polished fiber stub;

connector housing holds index-matching gel for low-loss signal transfer from cable to connector;

suitable for terminating fibers of breakout cables.

Appropriate tools for installation, cleaning and inspection/testing must be available from the manufacturer of the connector.

Features:

- Plug type:            IEC61754-4 SC

- Fiber type:           IEC 60793-2-10 G50/125 OM3

- Fiber buffering:      3mm

- Ferrule:              zirconia ceramic

- Housing:              plastic

- Colour:               aqua

- Insertion loss (IL):  typ. 0.1dB, max. 0.3dB

- IL increase after 500 matings:  typ. < 0.1dB

- Return loss (RL):     typ. 35dB, min. 22dB

- Re-termination:       up to 3x

- Operating temperature:  -25 to +70°C

- Cables boots included:  3mm

- RoHS compliant

Manufacturer:  Telegärtner

Type:          FFP

Part number:   J08080F0008