FO duplex patch cord LC-HD-D Uniboot, OM4, 10.0m

with high density LC duplex Uniboot connectors on both ends;

plugs have push-/pull latch for improved grip;

fiber polarity can be changed without using tools;

individual plugs have red or black ring for clear fiber identification and cable polarity (A-B/A-A).

Pre-assembled dust covers.

Colour of connectors and cable jacket as specified by ANSI/TIA.

Features:

Length:                 10.0m

Tensile strength:       100N

RoHS:                   compliant

Connector

- Type:                 IEC61754-20 LC

- Performance class:    grade M

- Ferrule:              zirconia ceramic

- Housing:              plastic

- Colour:               violet

- Insertion loss (IL):  typ. ≤ 0.2dB

- Return loss (RL):     ≥ 27dB

- Mating cycles:        ≥ 1000

Cable:

- Type:                 I-K(ZN)H

- Outer diameter:       ≤ 2.0mm

- Colour:               violet

Fiber:

- Type:                 IEC 60793-2-10 G50/125 OM4

- Insertion loss (IL):  ≤ 3.5dB/km @ 850nm

- Bandwidth:            ≥ 3500 MHzxkm @850nm

Manufacturer:      Telegärtner

Type:              FO patch cord LC-HD-D Uniboot

Part number:       L00875U0003